MI  MARKET INTELLIGENCE TOOLS   ·   marketintelligencetools.com/templates/

Root Cause Analysis Report

Fill each section in order. Do not skip to corrective actions before the root cause is confirmed. Italic text shows the worked example from Ridgeline Components; overwrite it with your own investigation.

Report #: ____________     Prepared by: ____________     Date: ____________

1. Problem Statement

Name the asset or process, the date and time, and the measured impact. “The machine broke” carries no data anyone can act on.

| FIELD | ENTRY |
| --- | --- |
| Asset / process | CNC station 4 |
| Date & time | Tuesday, 06:40–14:10 |
| Downtime duration | 7.5 hours |
| Cost impact | $22,400 customer return (scrap and freight) |
| Description | 58 of 340 brackets (17%) failed incoming inspection for exceeding a 0.010-inch tolerance on a mounting hole. |

2. Timeline of Events

Order every event chronologically from the last normal state through the failure.

| DATE / TIME | EVENT |
| --- | --- |
| 90 days before shipment | Locating pin due for calendar-based replacement; interval missed |
| Shipment day, 06:40 | First bracket of the run machined on CNC station 4 |
| 06:40–14:10 | 340 brackets produced across the shift |
| 14:10 | Batch shipped to the automotive supplier |
| +2 days | Incoming inspection rejects 58 units (17%) for tolerance |
| +3 days | Investigation opens; station log shows the pin at 93 days old |

3. Root Cause

Keep asking “why” until the answer stops producing a deeper “why.” A cause that still has a “why” behind it is a symptom, not the root.

Worked example: A pin replacement interval tracked by calendar date, on a station running above its typical load for six weeks, let the locating pin run 3 weeks past its rated 90-day interval. The worn pin let the part drift 0.008 inches per cycle, enough to push later units past the 0.010-inch tolerance.

Your root cause: ______________________________________________________________________

: ______________________________________________________________________

4. Contributing Factors

A contributing factor raised the odds or severity of the failure without causing it alone.

| # | FACTOR |
| --- | --- |
| 1 | The daily inspection checklist for CNC station 4 didn't include a fixture wear check. |
| 2 | PM tracking logged the pin by calendar date, not cycle count, so the overdue interval was never flagged automatically. |
| 3 |  |
| 4 |  |

5. Corrective & Preventive Actions

Assign every solution to a named owner with a due date. Track status in the companion Corrective Action Tracker (XLSX).

| ACTION | OWNER | DUE DATE |
| --- | --- | --- |
| Replace the locating pin on CNC station 4 | Maintenance lead | Same day |
| Switch PM tracking on all 4 CNC stations from calendar date to cycle count | Reliability engineer | 15 business days |
| Add a fixture wear check to the daily inspection checklist | Quality supervisor | 5 business days |

6. Verification Plan

Confirm the solution worked before closing the report, and log any follow-ups still open.

| VERIFICATION METHOD | CHECK DATE | FOLLOW-UP |
| --- | --- | --- |
| Measure the first 25 units of the next batch for tolerance | 30-day mark | 60-day review to confirm the fix held |

marketintelligencetools.com/templates/root-cause-analysis-report-template — pairs with the Corrective Action Tracker (XLSX)